How to add an (S)VGA driver to XFree86

Copyright (c) 1993, 1994 David E. Wexelblat <dwex@XFree86.org>

Issue 1.3 - May 29, 1994

1.  Introduction

Adding support for a new SVGA chipset to XFree86 is a challenging

project for someone who wants to learn more about hardware-level

programming.  It can be fraught with hazards (in particular, crashing

the machine is all too common).  But in the end, when the server comes

up and functions, it is immensely satisfying.

Adding support for an SVGA chipset does not change any of the basic

functioning of the server.  It is still a dumb 8-bit PseudoColor

server or 1-bit StaticGray server.  Adding support for new hardware

(e.g. accelerated chips) is a major undertaking, and is not anywhere

near formalized enough yet that it can be documented.

Nonetheless, the driver-level programming here is a good introduction.

And can well be the first step for adding support for an accelerated

chipset, as many are SVGA-supersets.  Writing an SVGA-level driver for

the chipset can provide a stable development platform for making use

of new features (in fact, this has been done for the S3, Cirrus, and

WD accelerated chipsets, for internal use as the accelerated servers

are developed for XFree86 2.0).

Now let's get down to it.  In addition to this documentation, a stub

driver has been provided.  This should provide you a complete

framework for your new driver.  Don't let the size of this document

persuade you that this is an overly difficult task.  A lot of work has

been put into making this document as close to complete as possible;

hence it should, in theory, be possible to use this as a cookbook, and

come out with a working driver when you reach the end.  I do advise

that you read it all the way through before starting.

2.  Getting Started

The first step in developing a new driver is to get the documentation

for your chipset.  I've included a list of vendor contact information

that I have collected so far (it's far from complete, so if you have

any that isn't on the list, please send it to me).  You need to obtain

the databook for the chipset.  Make sure that the person you speak to

is aware that you intend to do register-level programming (so they

don't send you the EE-style datasheet).  Ask for any example code, or

developer's kits, etc.  I've learned that at the SVGA level, in

general, a databook that lists and describes the registers is the most

you can hope to find.

If you are not familiar with VGA register-level programming, you

should get (and read!) a copy of Richard Ferraro's bible (see

references below).  The best way to understand what is happening in

the server is to study the workings of the monochrome server's

``generic'' server, and compare it with the documentation in Ferraro's

book (be aware that there are a few errors in the book).  You can find

the generic-VGA-register handling functions in the file ``vgaHW.c''.

Once you understand what's happening in the generic server, you should

study one or more of the existing SVGA drivers.  Obtain the databook

for a supported SVGA chipset, and study the documentation along with

the code.  When you have a good understanding of what that driver does

over and above the generic VGA, you will know what information you

need to obtain from the databook for the new chipset.  Once you have

this information, you are ready to begin work on your new driver.

3.  Directory Tree Structure

Here is an outline of the directory tree structure for the source

tree.  Only directories/files that are relevant to writing a driver

are presented.  The structure for the Link Kit is presented below.

xc/config/cf/

site.def

Local configuration customization

xf86site.def

XFree86 local configuration customization

xc/programs/Xserver/hw/xfree86/

The server source

common/

Files common to all of the server (XF86Config parser, I/O

device handlers, etc)

xf86.h

Contains the `ScrnInfoRec' data structure

xf86_Option.h

Contains option flags

compiler.h

Contains in-line assembler macros and utility functions

os-support/

OS-support layer

assyntax.h

Contains macro-ized assembler mnemonics

xf86_OSlib.h

OS-support includes, defines, and prototypes

LinkKit/

site.def.LK

Template for Link Kit site.def

vga256/

256-color VGA server directories

vga/

The generic VGA handling code

vga.h

Contains the `vgaVideoChipRec' and `vgaHWRec' data

structures

vgaHW.c

Contains the generic-VGA-register handling functions

vgaHWInit(), vgaHWSave() and vgaHWRestore().

drivers/

Contains the SVGA driver subdirectories.  Each contains an

Imakefile, a .c file for the driver, and a .s file for the

bank- switching functions.

vga2/

The monochrome vga server directories.  Most of the files are

linked from vga256, and the differences handled by

conditional compilation.

drivers/

The SVGA driver subdirectories.  The `generic' VGA driver

is also located here.

vga16/

The 16-color vga server directories.  Most of the files are

linked from vga256, and the differences handled by

conditional compilation.

drivers/

The SVGA driver subdirectories.

VGADriverDoc/

This documentation and the stub driver.

The Link Kit is usually installed in /usr/X11R6/lib/Server.  The Link

Kit contains everything that is needed to relink the server.  It is

possible to write a new driver and build a new server without having

even the server source installed.

Server/

site.def

Local configuration customization

include/

All of the include files listed under the `common' directory

above

drivers/

All of the SVGA drivers

vga2/

The SVGA driver subdirectories.

vga16/

The SVGA driver subdirectories.

vga256/

The SVGA driver subdirectories.

VGADriverDoc/

The directory with this documentation and the stub driver.

`vgaHW.c' is also copied here, for reference (it is not built

as part of the Link Kit).

4.  Setting Up The Build Information

This section describes the peripheral configuration and build steps

that must be performed to set up for your new driver.  The steps are

the same whether you are building from the source tree of from the

Link Kit; only the locations of the files is different.  Here are the

configuration steps that must be followed:

1. Choose the name for your driver subdirectory and data

structures.  Since the current driver scheme allows (in fact,

encourages) putting drivers for multiple related chipsets in a

single driver, it is usually best to use the vendor name, rather

than a chipset version.  The fact that older XFree86 drivers do

not follow this convention should not deter you from using it

now - most of that code was developed before the driver

interface had been made flexible and extensible.

For this documentation, we'll use chips from the SuperDuper

Chips vendor.  Hence, we'll use `sdc' for the name of the

driver.

2. Decide whether your driver will support the color server, the

monochrome server, or both.  For this documentation, we will

assume that both the color and monochrome servers will be

supported.  If you intend to support only the color server, the

steps for the monochrome server can be ignored.  If you intend

to support only the monochrome server, the steps for the color

server listed should be performed for the monochrome server, and

the monochrome steps ignored.  Most of the existing drivers

support only the color or both servers; the ``generic'' driver

is the only driver (currently) that supports just the monochrome

server.

3. Create your driver directories:

o  If you are working in the source tree, create the following

directories:

xc/programs/Xserver/hw/xfree86/vga256/drivers/sdc

xc/programs/Xserver/hw/xfree86/vga16/drivers/sdc

xc/programs/Xserver/hw/xfree86/vga2/drivers/sdc

o  If you are working in the Link Kit, create the following

directories:

/usr/X11R6/lib/Server/drivers/vga256/sdc

/usr/X11R6/lib/Server/drivers/vga16/sdc

/usr/X11R6/lib/Server/drivers/vga2/sdc

4. Set up the Imakefile parameters to cause your driver to be

built:

o  If you are working in the source tree:

a. Edit the file xc/config/cf/xfree86.cf, and add `sdc' to

the list for the definitions for `XF86Vga256Drivers',

`XF86Vga16Drivers' and `XF86Vga2Drivers'.  You should put

`sdc' just before `generic' in the list (i.e. second

last), to ensure that none of the other driver's probe

functions incorrectly detect the `sdc' chipset .

b. Edit the file xc/config/cf/xf86site.def, and add the same

entries in this file (this is just a comment that shows

the default values).

c. Edit the site.def.LK file in

xc/programs/Xserver/hw/xfree86/LinkKit/, and add the same

entries in this file.  This is the prototype `site.def'

file that will be installed in the Link Kit.

o  If you are working in the Link Kit, edit the file

/usr/X11R6/lib/Server/site.def, and add `sdc' to the

`XF86Vga256Drivers', `XF86Vga16Drivers' and `XF86Vga2Drivers'

definitions as described in (a) above.

5. Now copy the prototype files into your new directories:

o  If you are working in the source tree, copy the `stub' files

as follows (directories are below xc/programs/Xserver):

Imakefile.stub =>

hw/xfree86/vga256/drivers/sdc/Imakefile

stub_driver.c =>

hw/xfree86/vga256/drivers/sdc/sdc_driver.c

stub_bank.s =>

hw/xfree86/vga256/drivers/sdc/sdc_bank.s

Imakefile.stub =>

hw/xfree86/vga16/drivers/sdc/Imakefile (then edit this

Imakefile and make the changes described in the comments).

Imakefile.stub =>

hw/xfree86/vga2/drivers/sdc/Imakefile (then edit this

Imakefile and make the changes described in the comments).

o  If you are working in the Link Kit, copy the `stub' files as

follows:

Imakefile.stub =>

/usr/X11R6/lib/Server/drivers/vga256/sdc/Imakefile

stub_driver.c =>

/usr/X11R6/lib/Server/drivers/vga256/sdc/sdc_driver.c

stub_bank.s =>

/usr/X11R6/lib/Server/drivers/vga256/sdc/sdc_bank.s

Imakefile.stub =>

/usr/X11R6/lib/Server/drivers/vga16/sdc/Imakefile (then

edit this Imakefile and make the changes described in the

comments).

Imakefile.stub =>

/usr/X11R6/lib/Server/drivers/vga2/sdc/Imakefile (then

edit this Imakefile and make the changes described in the

comments).

6. Edit each of the files you've just copied, and replace `stub'

with `sdc' and `STUB' with `SDC' wherever they appear.

That's all the prep work needed.  Now it's time to work on the actual

driver.

5.  The Bank-Switching Functions

The normal VGA memory map is 64k starting at address 0xA0000.  To

access more than 64k of memory, SuperVGA chipsets implement ``bank

switching'' - the high-order address bits are used to select the bank

of memory in which operations will take place.  The size and number of

these banks varies, and will be spelled out in the chipset

documentation.  A chipset will have zero, one or two bank registers.

Likely the ONLY case of zero bank registers is a generic VGA, and

hence is not a concern.

Note that some of the newer chipsets (e.g. Trident 8900CL, Cirrus)

allow for a linear mapping of the video memory.  While using such a

scheme would improve the performance of the server, it is not

currently supported.  Hence there is no way to use such features for a

new chipset.

Most SVGA chipsets have two bank registers.  This is the most

desirable structure (if any banking structure can be called

``desirable''), because data can be moved from one area of the screen

to another with a simple `mov' instruction.  There are two forms of

dual-banking - one where the two bank operations define a read-only

bank and a write-only bank, and one with two read/write windows.  With

the first form, the entire SVGA memory window is used for both read a

write operations, and the two bank registers determine which bank is

actually used (e.g. ET3000, ET4000).  With the second form, the SVGA

memory window is split into two read/write banks, with each bank

pointer being used to control one window.  In this case, one window is

used for read operations and the other for write operations (e.g.

PVGA1/Western Digital, Cirrus).

A chipset that has a single bank register uses that one bank for both

read and write access.  This is problematic, because copying

information from one part of the screen to another requires that the

data be read in, stored, and then written out.  Fortunately, the

server is able to handle both one-bank and two-bank chipsets; the

determination of behavior is defined by an entry in the driver data

structure described below.

A driver requires that three assembly-language functions be written,

in the file `sdc_bank.s'.  These functions set the read bank -

SDCSetRead(), the write bank - SDCSetWrite(), and set both banks -

SDCSetReadWrite().  For a chipset with only one bank, all three will

be declared as entry points to the same function (see the ``tvga8900''

driver for an example).

The functions are fairly simple - the bank number is passed to the

function in register %al.  The function will shift, bitmask, etc -

whatever is required to put the bank number into the correct form -

and then write it to the correct I/O port.  For chipsets where the two

banks are read-only HERE and write-only, the SetReadWrite() function

will have to do this twice - once for each bank.  For chipsets with

two independent read/write windows, the SetReadWrite() function should

use the same bank as the SetWrite() function.

A special note - these functions MUST be written in the macroized

assembler format defined in the header file ``assyntax.h''.  This will

ensure that the correct assembler code will be generated, regardless

of OS.  This macroized format currently supports USL, GNU, and Intel

assembler formats.

That's all there is to the banking functions.  Usually the chipset

reference will give examples of this code; if not, it is not difficult

to figure out, especially using the other drivers as examples.

6.  The Driver Itself

Now it's time to get down to the real work - writing the major driver

functions in the files sdc_driver.c.  First, an overview of what the

responsibilities of the driver are:

1. Provide a chipset-descriptor data structure to the server.  This

data structure contains pointers to the driver functions and

some data-structure initialization as well.

2. Provide a driver-local data structure to hold the contents of

the chipset registers.  This data structure will contain a

generic part and a driver-specific part.  It is used to save the

initial chipset state, and is initialized by the driver to put

the chipset into different modes.

3. Provide an identification function that the server will call to

list the chipsets that the driver is capable of supporting.

4. Provide a probe function that will identify this chipset as

different from all others, and return a positive response if the

chipset this driver supports is installed, and a negative

response otherwise.

5. Provide a function to select dot-clocks available on the board.

6. Provide functions to save, restore, and initialize the driver-

local data structure.

7. Provide a function to set the starting address for display in

the video memory.  This implements the virtual-screen for the

server.

8. Perhaps provide a function for use during VT-switching.

9. Perhaps provide a function to check if each mode is suitable for

the chipset being used.

Before stepping through the driver file in detail, here are some

important issues:

1. If your driver supports both the color and monochrome servers,

you should take care of both cases in the same file.  Most

things are the same - you can differentiate between the two with

the MONOVGA #define.  If the 16 color server is supported, code

specific to it can be enabled with the XF86VGA16 #define.  In

most cases  it is sufficient to put the following near the top

of the stub_driver.c file:

#ifdef XF86VGA16

#define MONOVGA

#endif

2. The color server uses the SVGA's 8-bit packed-pixel mode.  The

monochrome and vga16 servers uses the VGA's 16-color mode (4

bit-planes).  Only one plane is enabled for the monochrome

server.

3. It is possible for you to define your monochrome driver so that

no bank-switching is done.  This is not particularly desirable,

as it yields only 64k of viewing area.

Keeping these things in mind, you need to find the registers from your

SVGA chipset that control the desired features.  In particular, regis-

ters that control:

1. Clock select bits.  The two low-order bits are part of the

standard Miscellaneous Output Register; most SVGA chipsets will

include 1 or 2 more bits, allowing the use of 8 or 16 discrete

clocks.

2. Bank selection.  The SVGA chipset will have one or two registers

that control read/write bank selection.

3. CRTC extensions.  The standard VGA registers don't have enough

bits to address large displays.  So the SVGA chipsets have

extension bits.

4. Interlaced mode.  Standard VGA does not support interlaced

displays.  So the SVGA chipset will have a bit somewhere to

control interlaced mode.  Some chipsets require additional

registers to be set up to control interlaced mode

5. Starting address.  The standard VGA only has 16 bits in which to

specify the starting address for the display.  This restricts

the screen size usable by the virtual screen feature.  The SVGA

chipset will usually provide one or more extension bits.

6. Lock registers.  Many SVGA chipset prevent modification of

extended registers unless the registers are first ``unlocked''.

You will need to disable protection of any registers you will

need for other purposes.

7. Any other facilities.  Some chipset may, for example, require

that certain bits be set before you can access extended VGA

memory (beyond the IBM-standard 256k).  Or other facilities;

read through all of the extended register descriptions and see

if anything important leaps out at you.

If you are fortunate, the chipset vendor will include in the databook

some tables of register settings for various BIOS modes.  You can

learn a lot about what manipulations you must do by looking at the

various BIOS modes.

6.1.  Multiple Chipsets And Options

It is possible, and in fact desirable, to have a single driver support

multiple chipsets from the same vendor.  If there are multiple

supported chipsets, then you would have a series of #define's for

them, and a variable `SDCchipset', which would be used throughout the

driver when distinctions must be made.  See the Trident and PVGA1/WD

drivers for examples (the Tseng ET3000 and ET4000 are counter-examples

- these were implemented before the driver interface allowed for

multiple chipsets, so this example should NOT be followed).  Note that

you should only distinguish versions when your driver needs to do

things differently for them.  For example, suppose the SDC driver

supports the SDC-1a, SDC-1b, and SDC-2 chipsets.  The -1a and -1b are

essentially the same, but different from the -2 chipset.  Your driver

should support the -1 and -2 chipsets, and not distinguish between the

-1a and -1a.  This will simplify things for the end user.

In cases where you want to give the user control of driver behavior,

or there are things that cannot be determined without user

intervention, you should use ``option'' flags.  Say that board vendors

that use the SDC chipsets have the option of providing 8 or 16 clocks.

There's no way you can determine this from the chipset probe, so you

provide an option flag to let the user select the behavior from the

XF86Config file.  The option flags are defined in the file

``xf86_option.h''.  You should look to see if there is already a flag

that can be reused.  If so, use it in your driver.  If not, add a new

#define, and define the string->symbol mapping in the table in that

file.  To see how option flags are used, look at the ET4000, PVGA1/WD,

and Trident drivers.

6.2.  Data Structures

Once you have an understanding of what is needed from the above

description, it is time to fill in the driver data structures.  First

we will deal with the `vgaSDCRec' structure.  This data structure is

the driver-local structure that holds the SVGA state information.  The

first entry in this data structure is ALWAYS `vgaHWRec std'.  This

piece holds the generic VGA portion of the information.  After that,

you will have one `unsigned char' field for each register that will be

manipulated by your driver.  That's all there is to this data

structure.

Next you must initialize the `SDC' structure (type `vgaVideoChipRec').

This is the global structure that identifies your driver to the

server.  Its name MUST be `SDC', in all caps - i.e. it must match the

directory name for your driver.  This is required so that the Link Kit

reconfiguration can identify all of the requisite directories and

global data structures.

The first section of this structure simply holds pointers to the

driver functions.

Next, you must initialize the information about how your chipset does

bank switching.  The following fields must be filled in:

1. ChipMapSize - the amount of memory that must be mapped into the

server's address space.  This is almost always 64k (from 0xA0000

to 0xAFFFF).  Some chipsets use a 128k map (from 0xA0000 to

0xBFFFF).  If your chipset gives an option, use the 64k window,

as a 128k window rules out using a Hercules or Monochrome

Display Adapter card with the SVGA.

2. ChipSegmentSize - the size of each bank within the ChipMapSize

window.  This is usually also 64k, however, some chipsets split

the mapped window into a read portion and a write portion (for

example the PVGA1/Western Digital chipsets).

3. ChipSegmentShift - the number of bits by which an address will

be shifted right to mask of the bank number.  This is log-base-2

of ChipSegmentSize.

4. ChipSegmentMask - a bitmask used to mask off the address within

a given bank.  This is (ChipSegmentSize-1).

5. ChipReadBottom,ChipReadTop - the addresses within the mapped

window in which read operations can be done.  Usually 0, and

64k, respectively, except for those chipset that have separate

read and write windows.

6. ChipWriteBottom,ChipWriteTop - same as above, for write

operations.

7. ChipUse2Banks - a boolean value for whether this chipset has one

or two bank registers.  This is used to set up the screen-to-

screen operations properly.

There are three more fields that must be filled in:

1. ChipInterlaceType - this is either VGA_NO_DIVIDE_VERT or

VGA_DIVIDE_VERT.  Some chipsets require that the vertical timing

numbers be divided in half for interlaced modes.  Setting this

flag will take care of that.

2. ChipOptionFlags - this should always be `{0,}' in the data

structure initialization.  This is a bitfield that contains the

Option flags that are valid for this driver.  The appropriate

bits are initialized at the end of the Probe function.

3. ChipRounding - this gets set to the multiple by which the

virtual width of the display must be rounded for the 256-color

server.  This value is usually 8, but may be 4 or 16 for some

chipsets.

6.3.  The Ident() function

The Ident() function is a very simple function.  The server will call

this function repeatedly, until a NULL is returned, when printing out

the list of configured drivers.  The Ident() function should return a

chipset name for a supported chipset.  The function is passed a number

which increments from 0 on each iteration.

6.4.  The ClockSelect() function

The ClockSelect() function is used during clock probing (i.e. when no

`Clocks' line is specified in the XF86Config file) to select the dot-

clock indicated by the number passed in the parameter.  The function

should set the chipset's clock-select bits according to the passed-in

number.  Two dummy values will be passed in as well (CLK_REG_SAVE,

CLK_SAVE_RESTORE).  When CLK_REG_SAVE is passed, the function should

save away copies of any registers that will be modified during clock

selection.  When CLK_REG_RESTORE is passed, the function should

restore these registers.  This ensure that the clock-probing cannot

corrupt registers.

This function should return FALSE if the passed-in index value is

invalid or if the clock can't be set for some reason.

6.5.  The Probe() function

The Probe() function is perhaps the most important, and perhaps the

least intuitive function in the driver.  The Probe function is

required to identify the chipset independent of all other chipsets.

If the user has specified a `Chipset' line in the XF86Config file,

this is a simple string comparison check.  Otherwise, you must use

some other technique to figure out what chipset is installed.  If you

are lucky, the chipset will have an identification mechanism

(ident/version registers, etc), and this will be documented in the

databook.  Otherwise, you will have to determine some scheme, using

the reference materials listed below.

The identification is often done by looking for particular patterns in

register, or for the existence of certain extended registers.  Or with

some boards/chipsets, the requisite information can be obtained by

reading the BIOS for certain signature strings.  The best advise is to

study the existing probe functions, and use the reference

documentation.  You must be certain that your probe is non-destructive

- if you modify a register, it must be saved before, and restored

after.

Once the chipset is successfully identified, the Probe() function must

do some other initializations:

1. If the user has not specified the `VideoRam' parameter in the

XF86Config file, the amount of installed memory must be

determined.

2. If the user has not specified the `Clocks' parameter in the

XF86Config file, the values for the available dot-clocks must be

determined.  This is done by calling the vgaGetClocks()

function, and passing it the number of clocks available and a

pointer to the ClockSelect() function.

3. It is recommended that the `maxClock' field of the server's

`vga256InfoRec' structure be filled in with the maximum dot-

clock rate allowed for this chipset (specified in KHz).  If this

is not filled in a probe time, a default (currently 90MHz) will

be used.

4. The `chipset' field of the server's `vga256InfoRec' structure

must be initialized to the name of the installed chipset.

5. If the driver will be used with the monochrome server, the

`bankedMono' field of the server's `vga256InfoRec' structure

must be set to indicate whether the monochrome driver supports

banking.

6. If any option flags are used by this driver, the

`ChipOptionFlags' structure in the `vgaVideoChipRec' must be

initialized with the allowed option flags using the OFLG_SET()

macro.

6.6.  The EnterLeave() function

The EnterLeave() function is called whenever the virtual console on

which the server runs is entered or left (for OSs without virtual

consoles, the function is called when the server starts and again when

it exits).  The purpose of this function is to enable and disable I/O

permissions (for OSs where such is required), and to unlock and relock

access to ``protected'' registers that the driver must manipulate.  It

is a fairly trivial function, and can be implemented by following the

comments in the stub driver.

6.7.  The Restore() function

The Restore() function is used for restoring a saved video state.

Note that `restore' is a bit of a misnomer - this function is used to

both restore a saved state and to install a new one created by the

server.  The Restore() function must complete the following actions:

1. Ensure that Bank 0 is selected, and that any other state

information required prior to writing out a new state has been

set up.

2. Call vgaHWRestore() to restore the generic VGA portion of the

state information.  This function is in the vgaHW.c file.

3. Restore the chipset-specific portion of the state information.

This may be done by simply writing out the register, or by doing

a read/modify/write cycle if only certain bits are to be

modified.  Be sure to note the comment in the sample driver

about how to handle clock-select bits.

6.8.  The Save() function

The Save() function is used to extract the initial video state

information when the server starts.  The Save() function must complete

the following actions:

1. Ensure that Bank 0 is selected.

2. Call vgaHWSave() to extract the generic VGA portion of the state

information.  This function is in the vgaHW.c file.

3. Extract the chipset-specific portion of the state information.

6.9.  The Init() function

The Init() function is the second most important function in the

driver (after the Probe() function).  It is used to initialize a data

structure for each of the defined display modes in the server.  This

function is required to initialize the entire `vgaSDCRec' data

structure with the information needed to put the SVGA chipset into the

required state.  The generic VGA portion of the structure is

initialized with a call to vgaHWInit() (also located in vgaHW.c).

Once the generic portion is initialized, the Init() function can

override any of the generic register initialization, if necessary.

All of the other fields are filled in with the correct initialization.

The information about the particular mode being initialized is passed

in the `mode' parameter, a pointer to a `DisplayModeRec' structure.

This can be dereferenced to determine the needed parameters.

If you only know how to initialize certain bits of the register, do

that here, and make sure that the Restore() function does a

read/modify/write to only manipulate those bits.  Again, refer to the

existing drivers for examples of what happens in this function.

6.10.  The Adjust() function

The Adjust() function is another fairly basic function.  It is called

whenever the server needs to adjust the start of the displayed part of

the video memory, due to scrolling of the virtual screen or when

changing the displayed resolution.  All it does is set the starting

address on the chipset to match the specified coordinate.  Follow the

comments in the stub driver for details on how to implement it.

6.11.  The ValidMode() function

The ValidMode() function is required.  It is used to check for any

chipset-dependent reasons why a graphics mode might not be valid.  It

gets called by higher levels of the code after the Probe() stage.  In

many cases no special checking will be required and this function will

simply return TRUE always.

6.12.  The SaveScreen() function

The SaveScreen() function is not needed by most chipsets.  This

function would only be required if the extended registers that your

driver needs will be modified when a synchronous reset is performed on

the SVGA chipset (your databook should tell you this).  If you do NOT

need this function, simply don't define it, and put `NoopDDA' in its

place in the vgaVideoChipRec structure initialization (NoopDDA is a

generic-use empty function).

If you DO need this function, it is fairly simple to do.  It will be

called twice - once before the reset, and again after.  It will be

passed a parameter of SS_START in the former case, and SS_FINISH in

the latter.  All that needs to be done is to save any registers that

will be affected by the reset into static variables on the SS_START

call, and then restore them on the SS_FINISH call.

6.13.  The GetMode() function

The GetMode() function is not used as of XFree86 1.3; its place in the

vgaVideoChipRec should be initialized to `NoopDDA'.

At some point in the future, this function will be used to enable the

server and/or a standalone program using the server's driver libraries

to do interactive video mode adjustments.  This function will read the

SVGA registers and fill in a DisplayModeRec structure with the current

video mode.

6.14.  The FbInit() function

The FbInit() function is required for drivers with accelerated

graphics support.  It is used to replace default cfb.banked functions

with accelerated chip-specific versions.  vga256LowlevFuncs is a

struct containing a list of functions which can be replaced.  This

struct defined in vga256.h.  Examples of FbInit() functions can be

found in the et4000, pvga1 and cirrus drivers.

If you do NOT need this function, simply don't define it, and put

`NoopDDA' in its place in the vgaVideoChipRec structure

initialization.

7.  Building The New Server

As in the setup work, the steps for building the server depend whether

you are working in the source tree or in the Link Kit.  Here are the

steps for the initial build after installing your new driver files:

o  If you are working in the source tree, follow these steps:

Go to xc/programs/Xserver, and enter `make Makefile', then `make

Makefiles depend all'

o  If you are working in the Link Kit, follow these steps:

1. Go to /usr/X11R6/lib/Server, and enter `./mkmf'

2. In the same directory, enter `make'

To rebuild the server after the initial build (e.g. after making

changes to your driver):

o  If you are working in the source tree, follow these steps:

1. Go to the appropriate drivers/ directory (e.g.,

xc/programs/Xserver/hw/xfree86/vga256/drivers), and enter

`make'.

2. Go to xc/programs/Xserver, and enter `make loadXF86_SVGA' (to

link the color server), `make loadXF86_VGA16' (to link the 16

color server) or `make loadXF86_Mono' (to link the mono

server).

o  If you are working in the Link Kit, follow these steps:

1. Go to the appropriate driver directory, and enter `make'.

2. Go to /usr/X11R6/lib/server, and enter `make loadXF86_SVGA'

(to link the color server) or `make loadXF86_VGA16' (to link

the 16 color server) or `make loadXF86_Mono' (to link the

mono server).

8.  Debugging

Debugging a new driver can be a painful experience, unfortunately.  It

is likely that incorrect programming of the SVGA chipset can lock up

your machine.  More likely, however, is that the display will be lost,

potentially requiring a reboot to correct.  It is HIGHLY recommended

that the server be run from an attached terminal or a network login.

This is the only rational way in which a debugger can be used on the

server.  Attempting to use multiple VTs for debugging is basically a

waste of time.

Because of the potential for locking up the machine, it is a VERY good

idea to remember to do a `sync' or two before starting the server.  In

addition, any unnecessary filesystems should be unmounted while the

debugging session is going on (to avoid having to run unnecessary

fsck's).

By default the server is built without debugging symbols.  The server

can grow VERY large with debugging enabled.  It is very simple to

rebuild your driver for debugging, though.  Do the following:

1. Go to the driver directory.

2. Edit the Makefile.  Look for the SECOND definition of

`CDEBUGFLAGS'.  Change this definition to

CDEBUGFLAGS = -g -DNO_INLINE

(this will enable debugging symbols and disable inlining of func-

tions, which can make single-stepping a nightmare).

3. Remove the `sdc_driver.o' file.

4. Now follow the steps above for rebuilding the server.

(Alternatively, instead of editing the Makefile, you can simply

do `make CDEBUGFLAGS="-g -DNO_INLINE"' after removing the old .o

file, then rebuild the server as described above).

This will give you a server with which you can set breakpoints in the

driver functions and single-step them.  If you are working in the

source tree, and just learning about SVGA programming, it may be

useful to rebuild vgaHW.c with debugging as well.

9.  Advice

I cannot stress this enough - study all available references, and the

existing code, until you understand what is happening.  Do this BEFORE

you begin writing a driver.  This will save you a massive amount of

headache.  Try to find a driver for a chipset that is similar to

yours, if possible.  Use this as an example, and perhaps derive your

driver from it.

Do not let the gloom-and-doom in the debugging section  discourage

you.  While you will probably have problems initially (I still do),

careful, deliberate debugging steps can bear fruit very quickly.  It

is likely that, given a good understanding of the chipset, a driver

can be written and debugged in a day or two.  For someone just

learning about this kind of programming, a week is more reasonable.

10.  Advanced Topics

Newer chipsets are getting into two advanced areas: programmable clock

generators, and accelerated capabilities (BitBlt, line drawing, HW

cursor).  These are new areas, and the formal interfaces to them are

not yet defined.  It is advised that you contact the XFree86 team and

get involved with the development/beta-testing team if you need to be

working in these areas.

11.  References

o  Programmer's Guide to the EGA and VGA Cards, 2nd ed.

Richard Ferraro

Addison-Wesley, 1990

ISBN 0-201-57025-4

(This is the bible of SVGA programming - it has a few errors, so

watch out).

o  vgadoc3.zip

Finn Thoegersen

(This is a collection of SVGA and other chipset documentation.

It is available on most MS-DOS/Windows related FTP archives,

including wuarchive.  It is DOS/BIOS oriented, but is still

extremely useful, especially for developing probe functions).

12.  Vendor Contact Information

ATI Technologies (VGA-Wonder, Mach8, Mach32)

33 Commerce Valley Drive East" Thornhill, Ontario

Canada L3T 7N6

(905) 882-2600 (sales)

(905) 882-2626 (tech support)

(905) 764-9404 (BBS)

(905) 882-0546 (fax)

Chips & Technologies

???

Cirrus Logic (SVGA, Accelerators - CL-GD5426)

3100 West Warren Ave.

Fremont, CA  94538

(510) 623-8300 (N. CA, USA)

(49) 8152-40084 (Germany)

(44) 0727-872424 (UK)

Genoa Systems (GVGA)

75 E. Trimble Road

San Jose, CA 95131

(408) 432-9090 (sales)

(408) 432-8324 (tech support)

Headland Technologies, Inc (Video-7 VGA 1024i, VRAM II)

46221 Landing Parkway

Fremont, CA  94538

(415) 623-7857

Oak Technology, Inc (OTI-067,OTI-077)

139 Kifer Ct.

Sunnyvale, CA 94086

(408) 737-0888

(408) 737-3838 (fax)

S3 (911, 924, 801

805, 928)/ (408) 980-5400

Trident Microsystems Inc (8800, 8900, 9000)

205 Ravendale Dr

Mountainside, CA 94043

(415) 691-9211

Tseng Labs Inc,

6 Terry Drive

Newtown, PA  18940

(215) 968-0502

Weitek (Power9000, 5186)

1060 E. Arques Ave,

Sunnyvale, CA  94086

(408) 738-5765

Western Digital

(714) 932-4900

$XConsortium: VGADriv.sgml,v 1.3 95/01/23 15:34:51 kaleb Exp $

Generated from XFree86: xc/programs/Xserver/hw/xfree86/doc/sgml/VGADriv.sgml,v 3.5 1995/01/28 16:02:34 dawes Exp $

$XFree86: xc/programs/Xserver/hw/xfree86/VGADriverDoc/VGADriver.Doc,v 3.13 1995/07/12 15:33:21 dawes Exp $